# Appendix 2

# Advantages/Disadvantages of parking control measures

| Type of control | Current use in Borough | Advantages | Disadvantages |
| --- | --- | --- | --- |
| Double yellow lines – no parking at any time | Numerous | - Provides for safety, movement and servicing requirements | - Cannot be parked on in non peak times – eg evenings/nightime - No exclusions |
| Single yellow lines – no parking at the times/days prescribed on nearby sign plates | Numerous | - Provides for safety, movement and servicing requirements at specified times, whilst providing for parking at other times. | - No selective parking or time limited parking in the parking periods |
| Yellow line – one hour in daytime | Near Thorpe Bay and Leigh Stations | - Largely self enforcing for preventing parking by train commuters. | - No-one can park for the one hour. - Long stay local commuters can move car and relocate during one hour ban - No way of controlling length of stay ( turn over ) during the free parking periods - No way of allowing any permit holders |
| As above but bays for permit holders for the one hour | None | - Largely self enforcing for preventing parking by train commuters. - Permit holders ( residents) can park at any time ( permits would need to be charged ) | - Only permit holders can park for the one hour - Long stay local commuters can move car and relocate during one hour ban - No way of controlling length of stay ( turn over ) during the free parking periods |
| Free parking bays for 1 or 2 hours with no return in 4 hours | Many including Leigh Broadway, London Road, Hamlet Court Road | - Some turn over occurs if enforcement is undertaken | - No income to offset any costs - Only serves the short stay requirement ( ie cannot have 3 or 4 hour stays for example) - Time consuming and difficult to enforce ( at least two visits for each vehicle) - No permit holders ( although these could be accommodated – but permit holders would pay whereas other park for free) |
| On street Pay and Display | Marine Parade, Southend etc | - Straight forward to enforce - Income to cover operational costs plus possible excess - Can cater for any period of parking - Control usage ( ie availability) by charging rates | - Need change available ( but this is a common form of parking charging) - Maintenance and cash collection costs/procedures - Initial costs |
| On street pay and Display bays shared with permit holders (ie shared use) | Town Centre Controlled Zone | - As above plus cater for permit holders | - As above - Resident permit holders have to compete with other users for space ( i.e. no dedicated bays) |